MERLIN ENERGY

Car Wash Step 3 — Questionnaire Restructure

Equipment Specifications Corrected with Market Research Data

Date: April 9, 2026

Source: Car_Wash_Step_3_Questionnaire.pages audit + live site (merlinenergy.net/wizard)

Reference Docs: ElCarWash_SiteModeling_Reference_v2.docx, PE-Backed-Car-Wash-Analysis-March-2026.docx

Status: CONFIDENTIAL — DRAFT

SSOT Pattern: Selection = the spec. If applicable, quantity follows as a conditional follow-up. Calculator: selection × qty × 0.746 for HP items. All HP ranges validated against manufacturer specs from Tommy, Sonny’s, Coleman Hanna, AVW, Motor City, WashX, AutoBrite, American Car Wash Solutions, Pro Bro, and others.

# Executive Summary

This document restructures the Merlin Energy car wash Step 3 questionnaire from 26 questions to 26 questions (net zero change) with the following improvements:

| Category | Detail |
| --- | --- |
| Questions | 26 total (same as live site). 3 killed, 3 added. |
| HP Corrections | Conveyor: 5/10/15 → 15/20/25 HP. Blowers: remove 5 HP option, keep 7.5/10/15 HP. Brushes: 3/5/7.5 → 1/1.5/2 HP (electric). Vacuum: add Central vs Individual type. |
| Bug Fixes | Blower and air compressor HP read as kW (33% overcount). Fix: × 0.746 conversion. Blower qty cap 6 → 14. |
| New Questions | peakCarsPerHour (duty cycle → BESS), waterHeaterType (gas vs electric = 30-50 kW swing), vacuumMotorSize reinstated with type selection. |
| Killed | avgTicketPrice (not consumed), membershipPlanActive (not consumed), naturalGasLine (replaced by fuel-type selections on HVAC and water heater). |
| Sequence | Operator tunnel walk: Facility → Kiosk → Conveyor → Pumps → Brushes → Reclaim → RO → Blowers → Vacuums → Air → EV → Building → Solar |
| Sources | Tommy Car Wash, Sonny’s CarWash Factory, Coleman Hanna, AVW, Motor City Wash Works, WashX Systems, AutoBrite, American Car Wash Solutions, Pro Bro, NCS/IDC, D&S, Broadway Equipment, CAT Pumps, JE Adams, Kleen-Rite |

# Section 01 — Facility & Operations (6 Questions)

What is this site and how hard do you run it? These questions establish the facility type, scale, operating pattern, and throughput. Wash Type gates all downstream questions.

| # | Field ID | Selection | +Qty | What It Drives | Status | Rationale | Sources |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | facilityType | Express Tunnel / Flex-Serve / Full-Serve / Self-Serve | — | Gates all downstream questions | OK | No change. Must lead because Self-Serve eliminates half the equipment questions. | Industry standard across all manufacturers |
| 2 | tunnelOrBayCount | 1 / 2 / 3 / 4 / 5 / 6 | — | Scale multiplier | FIX | FIX: Max raised from 4 to 6. Multi-tunnel PE-backed sites (Tommy’s, Mister) run 2-3+ tunnels per site. | Tommy Car Wash Systems; Mister Car Wash (500+ sites, multi-tunnel) |
| 3 | operatingHours | 10 / 12 / 14 / 16 hrs | — | Annual energy kWh | OK | No change. El Car Wash confirmed 7 AM-9 PM (14 hrs/day). | El Car Wash site modeling doc; elcarwash.com |
| 4 | daysPerWeek | 5 / 6 / 7 | — | Annual energy kWh | OK | No change. | — |
| 5 | dailyVehicles | Slider 50-600 | — | Revenue, water, chemical, duty cycle baseline | OK | No change. Slider max already extended to 600 per El Car Wash doc recommendation. | El Car Wash doc: 250-300 cars/day at Novi |
| 6 | peakCarsPerHour | Slider 20-80 | — | Duty cycle → peak demand → BESS shaving target | NEW | NEW. Saturday-noon peak determines simultaneous equipment runtime. At DTE $14/kW demand charge, each kW of peak = $168/yr. Calc: peak/tunnel_capacity = duty_cycle. Equipment kW × duty_cycle = peak demand. | El Car Wash doc: “Peak hour throughput (Saturday 10 AM-2 PM likely; sets demand charge)” listed as Should-Have data. Tommy: 1 car per belt foot per hour = 120 CPH on 120ft tunnel. |

# Section 02 — Equipment: Tunnel Walk (15 Questions)

Physical flow from kiosk entry to exit, then building systems. All HP ranges validated against multiple manufacturer specs. HP × 0.746 = kW conversion applied in calculator, not UI.

| # | Field ID | Selection | +Qty | What It Drives | Status | Rationale | Sources |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Zone A — Entry: Pay, then get pulled in |  |  |  |  |  |  |  |
| 7 | kioskControls | Yes / No | 1 / 2 / 3 kiosks | ~1 kW each | OK | Leads equipment section. Operator pays at kiosk before conveyor. Qty: sites have 1-3 kiosks depending on traffic. El Car Wash: 2-3 kiosks per site. | El Car Wash doc: “Payment kiosks: 2-3”; Tommy’s: kiosk + TommyClub bypass lane; Mister Car Wash: kiosk + LPR member lane |
| 8 | conveyorMotorSize | 15 HP / 20 HP / 25 HP / 30 HP | Chain (1) / Dual Belt (2) | 11-22 kW per motor | FIX | CORRECTED HP RANGE from 5/10/15 to 15/20/25/30. Old range was for mini-tunnels, not express. Dual belt systems have 2 motors = built-in redundancy (if 1 fails, other keeps running — site doesn’t shut down). Chain conveyors: 1 drive motor. Formula: qty × HP × 0.746 | Tommy: 15/20/25 HP dual belt drives; AutoBrite: “two motors, two gear boxes, two VFDs — backup operating system”; American Car Wash Solutions: 10 HP hydraulic power pack (chain); Industry guide: 30+ HP for tunnel conveyors |
| Zone B — Wash: Spray, scrub, rinse |  |  |  |  |  |  |  |
| 9 | highPressurePumps | 3 HP / 5 HP / 7.5 HP / 10 HP / 15 HP | 2-5 pumps | 15-56 kW total | FIX | CORRECTED to include 3 HP (self-serve scale) through 15 HP (express tunnel). Multiple pumps in centralized pump station. Formula: qty × HP × 0.746 | Tommy All-in-One Pump Station: 3/7.5/15 HP options; CAT Pumps 1050: 15 HP max, 10 GPM, 2200 PSI; American Car Wash Solutions: Dual + Single HP units in backroom; Kleen-Rite: CAT, Giant, General Pump for tunnel applications |
| 10 | brushMotors | 1 HP / 1.5 HP / 2 HP (electric) OR Hydraulic Pack: 5 / 10 / 15 HP | 6-18 brushes (electric) OR 1 pack | 13-40 kW total | FIX | CORRECTED from 3/5/7.5 HP per brush. Two drive types exist: (A) Electric gearmotors at 1-2 HP per brush — Tommy uses 1.5 HP on ALL brushes. (B) Central hydraulic power pack at 5-15 HP driving multiple brushes via hydraulic orbital motors. Question should ask: Electric or Hydraulic? Then size accordingly. | Tommy: 1.5 HP Sumitomo Cyclo 6000 gearmotors on all brushes; 3E: 5 HP hydraulic motors for mitters/side washers; NS Corp: 1 HP electric mitter; Econocraft: 2 HP electric; American Car Wash Solutions: 10 HP Power Pack in backroom; Motor City: 1.5 HP Dry N’ Shine drive |
| 11 | reclaimSystem | None / Partial / Full + VFD | — | 7-11 kW. Triggers RO. | OK | One system per site. Even multi-tunnel sites share one centralized reclaim system with underground tanks, oil-water separators, multi-stage filtration. Research confirms all express tunnels describe singular reclaim system. | NCS: “Reclaim up to 65-75% of water”; Starwash: “Reclaim up to 85% per wash cycle”; Rocket Express: “Reclaim and recycle 100% of wash water using PurWater Reclaim”; Magic Tunnel: “As little as 14 gallons fresh water per wash” |
| 12 | roSystem | Yes / No | — | 3-5 kW. Conditional on reclaim. | OK | Spot-free final rinse. Only shown if reclaim is not None. El Car Wash premium packages include spot-free. | NCS PurClean E3 Series; El Car Wash “Armor” package includes spot-free |
| Zone C — Dry: 50-60% of total draw. PRIMARY BESS TARGET. |  |  |  |  |  |  |  |
| 13 | blowerMotorSize | 7.5 HP / 10 HP / 15 HP | Slider 8-16 producers | 60-180 kW. THE load BESS shaves. | BUG | CORRECTED: Removed 5 HP (doesn’t exist for express producers) and 20 HP (only Wind Blade, rare). Standard: 10 HP or 15 HP per producer. Qty slider: 8-16 (was capped at 6 — express tunnels run 8-16 producers across 2-4 arches). BUG: Calculator reads HP value as kW = 33% overcount on every producer. A 10-blower × 15 HP site shows 150 kW but actual = 112 kW. Fix: × 0.746 in calc. | Tommy: 16 × 10 HP on 2-3 arches; American Car Wash Solutions: 8 × 15 HP on 2 arches; Sonny’s: 8-13 producers, up to 150 HP; Coleman Hanna: 10 HP std (15 HP opt), 7800 CFM per 10 HP; WashX: 7.5/10/15 HP; D&S: 10/15 HP; Pro Bro: 10/15 HP; Motor City Air One: 10 HP “similar output to competitor 15 HP”; Coleman Hanna Wind Blade: 20 HP (specialty) |
| Zone D — Exit: Off conveyor, vacuum lot, tire fill |  |  |  |  |  |  |  |
| 14 | vacuumStalls | Slider 0-20 | — | Stall count. Also parking proxy (×3). | OK | Stall count IS the quantity for energy calc. Also feeds parkingSpots fallback (stalls × 3) for DCFC ceiling. | El Car Wash: “Free vacuums” at all sites; El Car Wash doc: “10-14 stalls” estimated |
| 15 | vacuumType | Central System / Individual Stalls | Central: 1-3 motors; Individual: per stall | Central: 26-56 kW. Individual: 2-4 kW/stall. | FIX | REINSTATED and restructured. Two distinct systems exist: (A) Central vacuum: 1-3 large industrial blower motors (10-25 HP each) in mechanical room with underground piping to all stalls. Used by PE-backed express chains with free vacuums (El Car Wash, Mister, Tidal Wave). (B) Individual stall units: each stall has 2-3 small motors at 1.6 HP. Used by self-serve/coin-op. The operator knows which they have. HP follow-up depends on type selected. | El Car Wash doc: “Central vacuum system: 10-14 stalls, industrial, 35-50 HP, 26-37 kW”; JE Adams: 2-3 × 1.6 HP per individual unit; Kleen-Rite: “Electro Motors used in central vacuum systems”; IVS: push-button free-use models for express |
| 16 | airCompressorSize | 7.5 HP / 10 HP / 15 HP | 1 / 2 | 6-22 kW | BUG | HP RANGE CORRECTED from 5/7.5/10/15 to 7.5/10/15. BUG: Same HP-as-kW issue as blowers — fix: × 0.746. Powers pneumatic operations (tire fill, gate arms, chemical dispensing, conveyor controls). | American Car Wash Solutions: “Air Compressor 120 Gal 10 HP Motor”; El Car Wash doc: “Air compressor: 120-gal, 15 HP, 11 kW” |
| 17 | evChargingExisting | None / L2 / DCFC | 1-4 chargers | Baseline for DCFC rec in Step 5 | OK | EV chargers in parking lot. Count establishes baseline for Step 5 DCFC optimizer. El Car Wash currently has 0 but EV is a natural adjacency (3-5 min express dwell). | El Car Wash doc: “EV Charging: Not added. Add 4× L2 as Phase 2”; PE Analysis: “EV charging is natural adjacency” |
| Zone E — Building: Always-on loads, not tied to wash cycle |  |  |  |  |  |  |  |
| 18 | lightingType | LED / Fluorescent / Mixed | — | 5-15 kW | OK | LED light shows confirmed at El Car Wash (“LED light show tunnel experience”). LED = lower kW than fluorescent/mixed. | El Car Wash doc: “LED lighting + effects: Premium tunnel show, 12-15 kW”; American Car Wash Solutions: 9 LED lights + Disco arch |
| 19 | exteriorSignage | Standard / Large LED | 1-3 signs | 2-12 kW per sign | OK | El Car Wash uses branded Art Deco signage. Premium LED signs draw 8-12 kW; standard 2-5 kW. | El Car Wash doc: “Exterior signage: Branded, 8-12 kW” |
| 20 | hvacBuilding | None / Gas Furnace / Electric Heat Pump | — | Gas=0 kW. Electric=10-20 kW. | FIX | CHANGED from Yes/No to fuel-type selection. Gas furnace = 0 kW on electric meter. Electric heat pump = 10-20 kW that must be sized into BESS/solar. Calc needs to know what hits the meter, not just whether HVAC exists. | El Car Wash doc: “HVAC, office, IT, POS, LPR: 8-12 kW” |
| 21 | waterHeaterType | Gas / Electric / Tankless Electric / Heat Pump | — | Gas=0 kW. Electric=30-50 kW. Major swing. | NEW | NEW. Replaces naturalGasLine (Yes/No). Old question was “UI gate only, not consumed by calc.” Water heating is a 30-50 kW load that either hits the electric meter (electric/heat pump) or doesn’t (gas). This is a material difference for BESS/solar sizing. El Car Wash doc flags this as “Must-Have” missing data. | El Car Wash doc: “Water heating method per state: Electric = +30-50 kW. Gas = off electrical model. FL sites may differ from MI” — listed under Must-Have missing data |

# Section 03 — Solar & Site (5 Questions, All Industries)

Industry-agnostic section. What can I bolt on?

| # | Field ID | Selection | +Qty | What It Drives | Status | Rationale | Sources |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 22 | totalSiteAreaSqFt | Slider | — | Layout, DCFC feasibility | OK | No change. | El Car Wash doc: “50,000 sq ft” for Novi |
| 23 | roofType | Flat / Low-slope / Peaked | — | Usable solar % (70/55/40%) | OK | No change. | El Car Wash doc: “Metal/Concrete” |
| 24 | roofAreaSqFt | Slider | — | Solar kW capacity | OK | No change. | El Car Wash doc: “6,500 sq ft” for Novi |
| 25 | carportInterest | Yes / No | — | Carport solar (vacuum canopy) | OK | No change. El Car Wash vacuum canopies support panels = +54 kW potential. | El Car Wash doc: “Solar carports: Yes, Interested (+54 kW)” |
| 26 | parkingSpots | Slider | — | DCFC ceiling (20% rule) | OK | No change. | — |

# Appendix A — Equipment Market Research: Multi-Source HP Validation

## Conveyor Motors

| Equipment | Manufacturer / Source | HP Spec | Typical Qty | Key Detail |
| --- | --- | --- | --- | --- |
| Conveyor | Tommy Car Wash Systems | 15 / 20 / 25 HP | 2 (dual belt) | Dual belt = built-in backup. If 1 motor fails, other keeps site running. |
| Conveyor | AutoBrite Company | 15-25 HP each | 2 (dual belt) | “Two motors, two gear boxes, two sensors, two VFDs — backup operating system.” |
| Conveyor | American Car Wash Solutions | 10 HP (hydraulic pack) | 1 (chain) | 80ft tunnel package. Chain conveyor with hydraulic power pack. |
| Conveyor | Broadway Equipment | Hydraulic motor | 1 drive | “Reversible hydraulic system for a quick solution should a problem arise.” |
| Conveyor | Industry Guide (Alibaba/Car Wash Advisory) | 30+ HP | 1-2 | Guide for tunnel-scale conveyors. Smaller HP (5 HP) is for mini-tunnel/IBA. |
| Conveyor | NS Corporation | 5 HP hydraulic pack | 1 | 20ft mini-conveyor only. NOT express tunnel scale. |

## Blower / Dryer Producers

| Equipment | Manufacturer / Source | HP Spec | Typical Qty | Key Detail |
| --- | --- | --- | --- | --- |
| Blower | Tommy Car Wash Systems | 10 HP per producer | 16 on 2-3 arches | Full Express drying system. Also uses twin gas-fired heated blowers above arches. |
| Blower | American Car Wash Solutions (80ft) | 15 HP per producer | 8 on 2 arches | 80ft tunnel package. 120 HP total blower array. |
| Blower | Sonny’s The CarWash Factory | 10-15 HP per producer | 8-13 (expandable) | Blower Frame System: up to 13 producers and 150 HP total. |
| Blower | Coleman Hanna H.E.S.S. | 10 HP std, 15 HP opt | Up to 5 per arch | Each 10 HP produces 7800 CFM. “Performance similar to other 15 HP producers.” |
| Blower | Coleman Hanna Wind Blade | 20 HP | 1 (specialty) | Placed 4-5ft behind final rinse. Removes bulk water before blower arches. |
| Blower | WashX Systems | 7.5 / 10 / 15 HP | Up to 15-producer arch | 7.5 HP for side producers, 10/15 HP for top producers. |
| Blower | D&S Car Wash | 10 HP or 15 HP | Single and Dual configs | Premier Producers in left, right, center configurations. |
| Blower | Pro Bro Car Wash Systems | 10 HP or 15 HP | Per arch config | UHMW polymer body. Height-adjustable arches. |
| Blower | Motor City Wash Works (Air One) | 10 HP | Per arch config | “Similar air flow output as other dryers using 15HP motors.” 92% efficiency. |
| Blower | NCS / IDC | 10 HP or 15 HP | Per arch | 200 MPH wind speed. Smooth Air Technology. |
| Blower | Broadway Equipment | 45 HP | 1 (fixed center unit) | Fixed center distributor fan + 2 oscillating distributors. |

## Brush / Wrap / Mitter Motors

| Equipment | Manufacturer / Source | HP Spec | Typical Qty | Key Detail |
| --- | --- | --- | --- | --- |
| Brushes | Tommy Car Wash Systems | 1.5 HP (electric gearmotor) | All brushes + tire shine | Sumitomo Cyclo 6000. 3-phase, 230/460V, Class F insulation. |
| Brushes | Tommy Car Wash (Drive Unit) | 1.5 HP (electric) | Reverse Hugger, Z-Wrap | Drive unit contains motor, cage, bearings, chain, shaft coupler. |
| Brushes | 3E The Carwash Manufacturer | 5 HP (hydraulic) | Mitter + side washer | Parker TE0065-style hydraulic motor. Orbital design. |
| Brushes | NS Corporation | 1 HP (electric) | Mitter curtain | Front-to-rear top curtain with electric motor drive. |
| Brushes | Econocraft | 2 HP (electric) | Side-to-side mitter | 8-basket mitter with 2 HP electric motor. |
| Brushes | American Car Wash Solutions | 10 HP (hydraulic power pack) | Central pack for all brushes | Backroom power pack drives multiple brush hydraulic motors. |
| Brushes | Car Wash Consignment (used) | 2 HP (hydraulic power unit) | Per brush system | Hydraulic motor driven with trunnion mounted air cylinders. |

## Vacuum Systems

| Equipment | Manufacturer / Source | HP Spec | Typical Qty | Key Detail |
| --- | --- | --- | --- | --- |
| Vacuum | El Car Wash Site Modeling Doc | 35-50 HP total (central) | 10-14 stalls | Central vacuum system, industrial. 26-37 kW. |
| Vacuum | JE Adams 9200 Series | 2-3 × 1.6 HP per unit | Per stall unit | Individual stall vacuum. 120V. Coin-op or push-button free-use. |
| Vacuum | Kleen-Rite / Electro Motor | Per motor (in central system) | Multiple per central | “Electro Motors used in central vacuum systems.” |
| Vacuum | IVS (Industrial Vacuum Systems) | Varies by # of motors | Per unit | On/off push button models for free-use vacuum at express washes. |

## High-Pressure Pumps

| Equipment | Manufacturer / Source | HP Spec | Typical Qty | Key Detail |
| --- | --- | --- | --- | --- |
| HP Pumps | Tommy All-in-One Pump Station | 3 / 7.5 / 15 HP | Centralized station | Powers all water applications in wash tunnel from one compact package. |
| HP Pumps | CAT Pumps 1050 | 15 HP max | Per pump | 10 GPM, 2200 PSI. Triplex plunger. Forged brass manifold. |
| HP Pumps | CAT Pumps 310 | 3-5 HP | Per pump (self-serve) | Most popular for self-serve bays. Belt-driven, triplex. |
| HP Pumps | American Car Wash Solutions | Dual + Single units | Backroom config | Multiple HP units in centralized pump room. |

## Air Compressor

| Equipment | Manufacturer / Source | HP Spec | Typical Qty | Key Detail |
| --- | --- | --- | --- | --- |
| Air Compressor | American Car Wash Solutions (80ft) | 10 HP | 1 (120 gal tank) | Standard 80ft express tunnel package. Includes air compressor dryer. |
| Air Compressor | El Car Wash Site Modeling Doc | 15 HP | 1 (120 gal) | 11 kW. Standard sizing per doc. |

# Appendix B — Questions Removed and Why

| Removed | Reason |
| --- | --- |
| avgTicketPrice | Not consumed by energy calculator. Revenue ROI context does not feed BESS, solar, or generator sizing. Outage revenue impact is captured separately. |
| membershipPlanActive | Not consumed by energy calculator. Subscription model flag has no impact on equipment load profile or energy sizing. |
| naturalGasLine | Yes/No was “UI gate only, not consumed by calc” per audit. Replaced by fuel-type selections on hvacBuilding (None/Gas Furnace/Electric Heat Pump) and waterHeaterType (Gas/Electric/Tankless/Heat Pump), which the calc can actually consume. Gas = 0 kW on meter. Electric = 10-50 kW. Material difference. |
| tunnelLengthFt (proposed, then cut) | Brush count and conveyor HP are asked directly. Tunnel length is redundant — the load is fully determined by equipment spec + quantity questions. |
| chemicalSystem (proposed, then cut) | ~1 kW total across 8-12 peristaltic pumps. Noise-level load. Not worth adding a question for. |
| Billing section - 5 Qs (proposed, then cut) | Wizard derives bill from equipment inputs (kW × duty cycle × hours = kWh, + demand charges). Billing data is a calibration/validation layer, not a sizing input. Belongs in Step 4/5 verification, not Step 3 questionnaire. |

# Appendix C — Key Corrections from Design Session

Critical findings and corrections identified during the April 9, 2026 working session, with supporting rationale.

| Correction | Detail & Rationale |
| --- | --- |
| Conveyor HP range wrong | Old: 5/10/15 HP. Corrected: 15/20/25/30 HP. The 5/10/15 range was for mini-tunnels and IBAs, not express tunnels. Tommy sells 15/20/25 HP drives. Industry guide cites 30+ HP for tunnel conveyors. Dual belt systems provide built-in motor redundancy — if primary fails, backup motor keeps site running. |
| Blower HP options incorrect | Old: 5/10/15/20 HP. Corrected: 7.5/10/15 HP. 5 HP producers don’t exist for express tunnels (smallest market offering is 7.5 HP from WashX). 20 HP is specialty (Coleman Hanna Wind Blade only). Standard is 10 HP or 15 HP per producer. |
| Blower count severely capped | Old max: 6. Corrected: 8-16. Express tunnels run 8-16 producers across 2-4 arches. Tommy runs 16 per tunnel. Sonny’s supports up to 13. American Car Wash Solutions 80ft package has 8. Old cap of 6 caused silent data loss on every express tunnel site. |
| Blower HP read as kW | BUG: Calculator reads HP value as kW with no conversion. A 15 HP blower (11.2 kW actual) is counted as 15 kW = 33% overcount per producer. With 10 producers, that’s 38 kW of phantom load cascading through BESS sizing, demand savings, and payback calculations. Fix: apply × 0.746 in calculator. |
| Air compressor same bug | Same HP→kW bug as blowers. 15 HP compressor (11.2 kW) counted as 15 kW. Fix: × 0.746. |
| Brush motors wildly oversized | Old: 3/5/7.5 HP per brush. Corrected: 1/1.5/2 HP (electric). Tommy uses 1.5 HP gearmotors on ALL brushes. Many tunnels use hydraulic brush motors driven by a central 5-15 HP hydraulic power pack — the pack HP is what hits the electric meter, not per-brush HP. Question restructured to ask electric vs hydraulic. |
| Vacuum question was wrong | Original proposal killed vacuumMotorSize based on JE Adams individual stall data (1.6 HP each). This was INCORRECT for express tunnels. El Car Wash doc specifies “Central vacuum system: 35-50 HP, 26-37 kW.” PE-backed chains with free vacuums use central industrial systems, not individual coin-op stalls. Question reinstated with type selection (Central / Individual). |
| naturalGasLine was useless | Audit said “UI gate only, not consumed by calc.” Replaced with fuel-type selections on HVAC and water heater. The calc needs to know what hits the electric meter. Gas water heater = 0 kW. Electric water heater = 30-50 kW. That’s a material swing for BESS/solar sizing. El Car Wash doc lists water heating method as “Must-Have” missing data. |
| Ticket price and membership removed | avgTicketPrice and membershipPlanActive are not consumed by the energy calculator. Revenue ROI context doesn’t feed BESS, solar, or generator sizing. Outage impact is captured directly in a separate resilience question. |
| Billing section removed from Step 3 | The wizard derives bills from equipment inputs. Billing data (monthly kWh, demand charge rate) is a calibration cross-check, not a sizing input. Belongs in Step 4/5 as “validate our estimate”, not in the core questionnaire. |
| Peak cars/hour added | Duty cycle multiplier. Same equipment at 50 cars/hr (everything simultaneous) vs 25 cars/hr (gaps between cars) produces 100 kW different peak demand. At DTE’s $14/kW, that’s $1,400/mo or $16,800/yr in demand charges from duty cycle alone. Model currently guesses; this question eliminates the guess. |
| Water heater type added | Replaces naturalGasLine. A gas water heater means 30-50 kW that does NOT get sized into BESS/solar. An electric heater means that load must be included. El Car Wash doc: “Electric = +30-50 kW. Gas = off electrical model.” |
| SSOT pattern clarified | Selection = the spec (user picks “15 HP”, that IS the value). Quantity = optional follow-up (“How many?”) on the same card. Not two parallel sub-questions. Calculator does: selection × qty × conversion. |
| Reclaim: 1 per site confirmed | Research confirms all express tunnels describe a single centralized reclaim system per site. Even multi-tunnel sites share one system. Underground tanks, oil-water separators, multi-stage filtration. No quantity question needed. |
| Operator tunnel-walk sequence | Questions resequenced to match physical flow: Kiosk (pay) → Conveyor (get pulled in) → Pumps (spray) → Brushes (scrub) → Reclaim/RO (rinse) → Blowers (dry) → Vacuums (post-wash) → Building (always-on). Operators mentally walk their facility entry-to-exit. Previous sequence jumped between zones. |

# Appendix D — Why Peak Cars/Hour Is the Highest-Value New Question

The equipment questions tell you WHAT’s installed. Peak cars/hour tells you how hard it all runs at the same time. That’s the duty cycle — and it directly determines the demand spike that BESS shaves.

## Scenario 1: Same Site, Two Time Windows

Same El Car Wash Novi tunnel. Same 12 blowers at 15 HP. The utility meter catches the highest 15-minute peak in the billing period. That one Saturday noon spike sets the demand charge for the entire month.

| Equipment | Saturday Noon (50 cars/hr) | Tuesday Morning (20 cars/hr) |
| --- | --- | --- |
| Blowers (12×15 HP) | All 12 continuous, no gaps = 134 kW | Idle 60% between cars = 54 kW |
| HP Pumps (4×10 HP) | Continuous cycling = 30 kW | Intermittent = 12 kW |
| Conveyor | 30 kW (always on) | 30 kW (always on) |
| Brushes (14×1.5 HP) | Continuous = 16 kW | Intermittent = 6 kW |
| Peak demand reading | ~310 kW | ~200 kW |
| Demand charge (DTE $14/kW) | $4,340/mo | $2,800/mo |
| DIFFERENCE |  | $1,540/mo = $18,480/yr |

Without peak cars/hour, the model takes daily vehicles (300) ÷ operating hours (14) = ~21 cars/hr average and assumes that’s the peak. Express tunnels are wildly spikey — the model can’t tell the difference without asking.

## Scenario 2: Two Sites, Same 300 Cars/Day, Different Peak Profiles

This is the scenario that kills the “just divide daily by hours” assumption. Membership-heavy sites have flat curves. Walk-in-heavy sites spike 2–3× on Saturday.

|  | Site A: Membership-Heavy (70%) | Site B: Walk-In Heavy (30%) |
| --- | --- | --- |
| Daily cars | 300 | 300 |
| Member % | 70% (unlimited plan) | 30% |
| Traffic pattern | Members spread across week, wash 2-3x on random days | Walk-ins cluster weekends and lunch hours |
| Peak hour | 30 cars/hr (flat curve, trickle in) | 50 cars/hr (Saturday noon crush) |
| Off-peak hours | Still 20-25/hr (members come anytime) | Down to 8-10/hr (Tuesday morning dead) |

Same equipment, same tunnel — demand meter sees two completely different sites:

| Load | Site A (30/hr peak) | Site B (50/hr peak) | Why |
| --- | --- | --- | --- |
| Blowers (12×15 HP) | 60% duty = 80 kW | 100% duty = 134 kW | At 30/hr, gaps between cars. Blowers idle. At 50/hr, continuous. |
| HP Pumps (4×10 HP) | 60% duty = 18 kW | 100% duty = 30 kW | Pumps cycle down between cars |
| Brushes (14×1.5 HP) | 60% duty = 9 kW | 100% duty = 16 kW | Brushes engage per car |
| Conveyor | 30 kW (always on) | 30 kW (always on) | Runs regardless of car count |
| Vacuums, HVAC, lighting | 62 kW (always on) | 62 kW (always on) | Base load, not car-dependent |
| PEAK DEMAND | ~199 kW | ~272 kW |  |
| Demand charge ($14/kW) | $2,786/mo | $3,808/mo |  |
| Annual demand charges | $33,432 | $45,696 |  |
| DIFFERENCE |  |  | $12,264/yr — same 300 cars, same revenue |

## BESS Sizing Divergence: Same Cars, Completely Different Investment

The payback is similar but the system size, investment, and absolute dollar savings are completely different. If the model assumes both sites are the same because they both do 300 cars/day, it either oversizes Site A’s BESS (wasting $75K in capital) or undersizes Site B’s (leaving $12K/yr on the table).

| BESS Decision | Site A (30/hr peak) | Site B (50/hr peak) |
| --- | --- | --- |
| Peak to shave | 199 kW → target 150 kW = shave 49 kW | 272 kW → target 150 kW = shave 122 kW |
| BESS size needed | ~100 kWh (small) | ~250 kWh (2.5× larger) |
| BESS cost (after ITC) | ~$50K | ~$125K |
| Annual demand savings | ~$8,200 | ~$20,500 |
| Payback | 6.1 years | 6.1 years |
| CAPITAL DIFFERENCE |  | $75K — wasted if you size both the same |

How the Calculator Uses This Input:

duty_cycle = peakCarsPerHour / tunnel_rated_capacity

peak_kW = each equipment kW × duty_cycle (variable loads) + always-on loads (conveyor, lighting, HVAC)

BESS_target = peak_kW – desired_threshold

monthly_demand_savings = shaved_kW × demand_rate ($/kW)

Conclusion: Site A’s peak is 43% above the daily average. Site B’s peak is 138% above the daily average. The model currently cannot tell these two sites apart. One slider question (the operator knows their Saturday bragging stat) eliminates a $12K–$18K/yr sizing error. That makes it the highest-value question in the entire questionnaire per dollar of accuracy it delivers.

# Appendix E — PE-Backed Car Wash Chains Referenced

Equipment specs validated against operations at these top US express tunnel chains. Source: PE-Backed-Car-Wash-Analysis-March-2026.docx

| Chain | PE Sponsor | Sites | Relevance |
| --- | --- | --- | --- |
| Mister Car Wash | Leonard Green & Partners | 500+ | Largest US chain. 74% revenue from Unlimited Wash Club. Multi-tunnel sites. Going private at $3.1B EV. |
| Whistle Express (+ Take 5) | Oaktree Capital | 530+ | Largest express operator. Integrating 380 Take 5 sites. Active rebranding. Equipment standardization across portfolio. |
| Quick Quack Car Wash | KKR + Seidler Equity | 275+ | 1M+ unlimited members. Patented anti-collision tech. KKR $850M minority stake. |
| Tidal Wave Auto Spa | Golden Gate Capital | 300+ | Founder-led. Premium positioning. Proprietary tech and cleaning products. 30 states. |
| El Car Wash | Warburg Pincus | 90+ | Reference client. FL + MI. 375K+ members. Merlin reference site: Novi MI. $8.6M annual energy spend. |
| Spotless Brands | Access Holdings + Wafra | 200+ | 6 brands. ~$200M EBITDA. Exploring ~$3B sale. $600M raised 2023. |
| Tommy’s Express | Franchise (PE at franchisee level) | 200+ | Largest US car wash franchise. Standardized equipment specs = standardized energy profiles. Key equipment data source for this document. |